The University of British Columbia

Mini Project 2

ELEC 301 – Electronic Circuits

Bryan Zhang 69238335 10-24-2022

Contents

Part 1	2

a)	2N3904 datasheet lookup	2

b)	2N3904 characteristics	2

Comparing Measured value to datasheet values	5

c)	Biasing	5

Biassing from measured plot (i)	5

Biassing using the 1/3 rule (ii)	6

Using standard resistors (iii)	7

d)	2N2222A and 2N4401 d.c operating points (iv)	7

Part 2	8

a)	2N3904 & 2N4401 Common Emitter Amplifier	8

b)	Midband Voltage Transfer Curve	11

c)	Input Impedance at midband	12

d)	Output Impedance at midband	12

e)	Transistor selection	13

Part 3	13

a)	2N2222A Common Base Amplifier poles & zeros	13

b)	Voltage Transfer Curve	16

c)	Midband Input Impedance	16

d)	Midband Output Impedance	16

References	17

Appendix	18

# Part 1

## 2N3904 datasheet lookup

The datasheet values for the 2N3904 transistor at VCE=10V, IC=1mA, f=1kHz, and T=1kHz are shown below.

| Symbol | Description | Minimum | Maximum |
| --- | --- | --- | --- |
| hfe=β | DC Current Gain | 100 | 400 |
| hie=rπ | Input Impedance | 1.0 kΩ | 10 kΩ |
| hoe=1r0 | Output Admittance | 1.0 µmhos | 40 µmhos |

## 2N3904 characteristics

Here is the circuit used to obtain the graph of Ib vs VBE. I set VCE=5V and then sweep VBE from 0 to 0.7V in 0.01V increments.

Figure 1: IB vs VBE graph

To find IC vs VCE with IB as the varying parameter the circuit below is simulated. VCE is swept from 0-6V in 20mV increments and IB is swept from 1-10µA in 0.5µA increments.

Figure 2: IC vs VCE with varying IB

Each line represents a different characteristic, with the bottom line being when IB is 1 µA and the top line when IB is 10 µA. The ‘a’ cursor is set to the value of VCE=5V and the ‘c’ cursor is set to the value of IC = 1mA. The intersection of the 2 cursors falls on the 3rd line from the top, so therefore

IB=10-3∙0.5μA=8.5μA.

yieldsβ=ICIB=1mA8.5μA=118

gm=ICVT=1mA25mV=0.04S

rπ=βgm=2950Ω

Finally, we can find IC vs VCE with varying VBE by simulating the below circuit. VCE is the primary and is swept from 0-6V in 20mV increments, and VBE is the secondary swept from 0.55-0.70V in 0.01V increments.

Figure 3: IC vs VCE with varying VBE

Observe that at IC=1mA at the intersected curve corresponds to VBE = 0.65V. The slope of the top curve can be calculated as m=c-da-b where a, b, c, d are the cursor positions.

m=c-da-b=251.9∙10-6A4V=6.30∙10-5S

An equation of the line can be found using point-slope form y-y1=m(x-x1). The Xb, Yd point is selected but any of the two points may be used.

IC-6.356∙10-3=6.30∙10-5(VCE-1)

Now the early voltage can be found at the x-intercept when IC=0:

-6.356∙10-3=6.30∙10-5VA-1yieldsVA=-100=100V

r0=VAIC=1001mA=100kΩ

### Comparing Measured value to datasheet values

The measured values of β, rπ, and 1/r0 all fall in between the minimum and maximum value found in the data sheet for the 2N3904 transistor.

## Biasing

### Biassing from measured plot (i)

The measured parameters from the curves are β=118 and VBE = 0.65V.

In addition, VCC = 15V, IC = 1mA, VCE < 4V, RE=RC/2. The other currents may be determined as follows:

IB=ICβ=1mA118=8.47μA

IE=IB+IC=8.47μA+1mA=1.0084mA

Next RC and RE can be determined by doing KVL from VCC to ground across the collector-emitter junction. VCE can be set to 3V since it is less than 4V.

VCC=ICRC+IERE+VCE=ICRC+12IERC+VCE

yieldsRC=VCC-VCEIC+12IE=15-31mA+12∙1.0084mA=7978Ω

RE=RC2=3989Ω

Now the voltage of the emitter, collector, and base can be calculated.

VE=IERE=1.0084mA∙3989Ω=4.022V

VC=VE+VCE=4.022V+3V=7.022V

VB=VE+VBE=4.012V+0.65V=4.662V

The following 2 equations can be used to determine the values of RB1 and RB2.

KCL:VCC-VBRB1=IB+VBRB2

KVL: VCCRB2RB1+RB2-IB(RB1|RB2-VB=0

These equations do not provide a unique solution, so I arbitrarily chose RB1=10kΩ which gives

RB2=4.55 kΩ.

| RC | RE | RB1 | RB2 |
| --- | --- | --- | --- |
| 7978Ω | 3989Ω | 10kΩ | 4.55kΩ |

Here are the results of the simulation for the d.c operating point for the 2N3904.

| IC | IB | IE | VB | VC | VE |
| --- | --- | --- | --- | --- | --- |
| 998.6µA | 8.590 µA | 1.007mA | 4.664V | 7.034V | 4.017V |

### Biassing using the 1/3 rule (ii)

I will use the first version of the 1/3 rule which states that:

VB=13VCC=5V, VC=23VCC=10V, I1=IEβ

Using β=118 and VBE = 0.65V we find that VE=VB-VBE=4.35V

Subsequently the currents are calculated

IB=ICβ=1mA118=8.47μA

IE=IB+IC=8.47μA+1mA=1.0084mA

I1=IEβ=1.0084mA118=92.83μA

I2=I1-IB=92.83μA-8.47μA=84.36μA

Finally, the resistances are calculated

RC=VCC-VCIC=5kΩ

RE=VEIE=4.35V1.0084mA=4.314kΩ

RB1=VCC-VBI1=10V92.83μA=107.72kΩ

RB2=VBI2=5V84.36μA=59.27kΩ

The d.c operating point values of the biased circuit using the 1/3 rule is shown below.

| IC | IB | IE | VB | VC | VE |
| --- | --- | --- | --- | --- | --- |
| 1.001mA | 8.431µA | 1.010mA | 5.002V | 9.994V | 4.356V |

### Using standard resistors (iii)

Replace the resistances calculated using the 1/3 rule with standard resistors.

RC=5.1kΩ, RE=4.3kΩ, RB1=110kΩ, RB2=62kΩ

| IC | IB | IE | VB | VC | VE |
| --- | --- | --- | --- | --- | --- |
| 1.020mA | 8.575µA | 1.028mA | 5.067V | 9.801V | 4.420V |

Every method biases the circuit correctly however using the 1/3 rule is much faster and still provides accurate results while allowing versatility when selecting the resistances.

## 2N2222A and 2N4401 d.c operating points (iv)

Using the same standard resistance values, we replace the 2N3904 with the 2N2222A and 2N4401 and measure their d.c operating points.

|  | IC | IB | IE | VB | VC | VE |
| --- | --- | --- | --- | --- | --- | --- |
| 2N3904 | 1.020mA | 8.575µA | 1.028mA | 5.067V | 9.801V | 4.420V |
| 2N222A | 1.055mA | 6.285µA | 1.061mA | 5.158V | 9.629V | 4.556V |
| 2N4401 | 1.035mA | 6.971µA | 1.042mA | 5.131V | 9.731V | 4.472V |

The BJTs are still biased properly in the active region. Therefore the 1/3 rule is a valid method of biasing BJTs even with different β values.

# Part 2

## 2N3904 & 2N4401 Common Emitter Amplifier

Here is the biased common emitter amplifier using the 2N3904.

The magnitude and phase(degrees) bode plots are included in the appendix simulated from 1mHz to 100GHz. Calculate Cπ and Cµ using the d.c operating point gm=0.04S and VCB = 5V and the SPICE model parameters of the 2N3904.

Cπ=2∙CJE+TF∙gm=2∙4.5pF+400ps∙0.04=25pF

Cμ=CJC1+VBVJCMJC=3.5pF(1+50.75)0.330≈2pF

The complete small-signal model is created for the 2N3904. Recall rπ = 2950Ω and β=118 from part 1.

Applying Miller’s theorem and Thevenin equivalent for the CE amplifier like in the course notes can yield the zeros and poles. There are two coupling capacitors so it is known that there will be 2 low frequency zeros at zero. This is the Low Frequency Small-Signal Model.

fLz1=fLz2=0 Hz

fLz3=12πRECE=3.70 Hz

fLp2=12πRC+RLCC2=1.56 Hz

fLp1=12πRs+RBB||rπ+(1+β)RECC1=432 mHz

fLp3=12πCErπ+RBB||RS1+β||RE=635 Hz

This is the High Frequency Small-Signal Model.

fHp1=12π(RBB||rπ)||RS(cπ+gm∙RL||RC)=15.5 MHz

fHp2=12πRC||RLcμ=34.9 MHz

Here are the locations of the poles and zeros estimated using linear approximation.

This table summarizes the calculated and graphically measured locations of the poles and zeros in Hz.

| 2N3904 | fLz1 | fLz2 | fLp1 | fLp2 | fLz3 | fLp3 | fHp1 | fHp2 | fHz1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Calculated | 0 | 0 | 0.432 | 1.56 | 3.70 | 635 | 15.5M | 34.9M | ∞ |
| Measured | 0 | 0 | 0.400 | 2.2 | 4 | 600 | 10M | 600M | 5G |
| % error | 0 | 0 | 8% | 29.1% | 7.5% | 5.83% | 55% | 94.1% |  |

The graphically measured values are quite like the calculated values other than the high frequency poles. There was also a high frequency zero in the simulated result.

Now replace the 2N3904 with the 2N4401. The magnitude and phase bode plots simulated from 1mHz to 100GHz for the 2N4401 are included in the appendix. Recalculate the location of the poles and zeros the same way as done before, except with new parameters for the parasitic capacitances of the 2N4401, and then recalculate β and rπ. β = 148, rπ = 3586Ω. Cπ=67.28pF, Cµ=5.2pF.

Then, graphically measure the poles/zeros with linear approximation and compare with the calculated.

Here are the calculated and graphically measured frequencies in Hz for the 2N4401.

| 2N4401 | fLz1 | fLz2 | fLp1 | fLp2 | fLz3 | fLp3 | fHp1 | fHp2 | fHz1 | fHz2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Calculated | 0 | 0 | 0.426 | 1.56 | 3.70 | 655 | 5.19M | 11.9M | ∞ | ∞ |
| Measured | 0 | 0 | 0.390 | 2 | 3.40 | 600 | 4M | 190M | 1G | 10G |
| % error | 0 | 0 | 9.23% | 22% | 8.82% | 9.17% | 29.8% | 93.7% |  |  |

Again, the graphically measured values are quite like the calculated values other then the high frequency poles. There are also two a high frequency zero in the simulated result.

## Midband Voltage Transfer Curve

A midband frequency of 100kHz is selected for this test. The input voltage source amplitude is varied from 0-0.3V.

The non-linear behavior starts to happen around 0.1V for both transistors.

## Midband Input Impedance

Here is the circuit at midband. CE will be shorted, and so the emitter will be grounded.

The input impedance for the 2N3904 can be calculated as Zin=RBB||rπ=2745Ω

The input impedance for the 2N4401 is Zin=RBB||rπ=3289Ω

To measure the input impedance set the AC source to 100kHz with 1V amplitude and then use the AC multimeter to measure the input current and base voltage.

Zin=VBIin=6.12mV1.755μA=3487Ω 2N3904

Zin=VBIin=6.14mV1.379μA=4453Ω (2N4401)

## Midband Output Impedance

At the midband the impedance seen at the output impedance is simply RC. A 10mV, 10kHz source is used.

Zout=RC=5.1kΩ

The output impedance is measured by

Zout=VoIout=570.1mV111.8μA=5.1kΩ (2N3904)

Zout=VoIout=592.4mV116.2μA=5.1kΩ (2N4401)

## Transistor selection

The 2N3904 has a larger bandwidth than the 2N4401 with other characteristics remaining similar. For an amplifier a wider bandwidth gives the best performance, so the 2N3904 should be selected.

# Part 3

## 2N2222A Common Base Amplifier poles & zeros

Calculate cπ=75.7pF and cµ=7.76pF with the new circuit parameters. The d.c operating point calculated in part 1 for the 2N2222A can be used to calculate β = 168, gm = 0.0422, and rπ = 3981Ω. There are 2 zeros at zero due to the coupling capacitors. We find the locations of the remaining poles/zeros using formulas taught in the class.

fLz1=fLz2=0 Hz

fLz3=12πRBBCB=0.401 Hz

fLp1=12π(RBB||(rπ+(1+β)RE))CB=0.423 Hz

fLp2=12πRC+RLCC2=1.56 Hz

fLp3=12πrπ1+β||RE+RSCC1=217 Hz

fHp1=12πRC||RLcμ=8.04 MHz

fHp2=12πrπ1+β|RE|RScπ=132 MHz

Approximate the locations using linear approximation. There must be a low frequency pole and zero that occur in the same location (as shown in the calculations), these are marked in the table by *.

Here are the calculated and approximated values for the locations of the poles in Hz. The cusp at ~1GHz may be caused by multiple zeros or a complex pole.

| 2N2222A | fLz1 | fLz2 | fLz3 | fLp1 | fLp2 | fLp3 | fHp1 | fHp2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Calculated | 0 | 0 | 0.401 | 0.423 | 1.56 | 217 | 8.04M | 132M |
| Measured | 0 | 0 | 0.401* | 0.423* | 1.80 | 220 | 8M | 130M |
| % error | 0 | 0 | 0 | 0 | 13.3% | 1.36% | 0.5% | 1.54% |

## Voltage Transfer Curve

Use a 1kHz source and vary the amplitude. It starts to become non-linear at around Vs = 0.1V.

## Midband Input Impedance

Calculate the input impedance as Zin=RE||11+βrπ=23.4Ω

To measure the input impedance, short the source resistance and apply a source with 10mV amplitude and 100kHz frequency. Use the AC RMS multimeter to measure the emitter voltage and input current.

Zin=VEIin=1.011mV40.09μA=25.2Ω

## Midband Output Impedance

The output impedance is simply Zout=RC=5.1kΩ

Zout=VCIin=70.36mV13.97μA=5.04kΩ

# References

- ELEC 301 Course Notes

- A. Sedra and K. Smith, "Microelectronic Circuits,"5 th (or higher) Ed., Oxford University Press, New York

- CircuitMaker™ User’s Manual

- 2N3904 datasheet [https://datasheetspdf.com/pdf-file/1114626/Motorola/2N3904/1]

- Standard Resistor and Capacitor Values

# Appendix

Part 2a:

2N3904 magnitude/phase bode plot:

2N4401 magnitude/phase bode plot: