The University of British Columbia

Mini Project 3

ELEC 301 – Electronic Circuits

Bryan Zhang 69238335 11-16-2022

Contents

Part 1 The Cascode Amplifier	2

A.	DC operating point	2

B.	Bode plots and estimated 3dB locations	4

C.	Voltage Transfer curve	5

D.	Input/Output Impedance measurement	6

Discussion	6

Part 2 Cascaded Amplifiers	6

A.	Biasing	6

B.	Measuring Input/Output Impedance & Midband Gain	8

C.	Bode Plots	10

Discussion	11

Part 3 Cascaded Amplifiers	11

A.	Bode plots	11

B.	Calculating differential gain and fH3dB	12

C.	Voltage Transfer curve	13

Discussion	13

Part 4 Cascaded Amplifiers	14

A.	Differential Output	14

B.	Sine wave Saturation	15

C.	Square wave Saturation	15

Discussion	15

References	16

Appendix	17

# Part 1 The Cascode Amplifier

## DC operating point

Looking at the CircuitMaker SPICE model, the 2N3904 β is found to be 300 (ideal maximum forward beta). Assume that VT=0.025V and VBE=0.7V. The cascode amplifier in d.c mode is shown to the left. Use the ¼ rule to bias the cascode amplifier.

VC2=34VCC=15V		VE2=VC1=12VCC=10V

VE1=14VCC=5V		VB1=VE1+VBE=5.7V

VB2=VE2+VBE=10.7V

At midband the value of Rout=RC=2.5kΩ±250Ω. Set RC to the closest standard value of 2.4kΩ and compute all currents.

IC2=VCC-VC2RC=2.083mA		IB2=IC2β=6.944μA		IE2=IC1=1+β∙IB2=2.09mA

IB1=IC1β=6.968μA			IE1=1+β∙IB1=2.097mA		I1=IE1β=121.1μA

I2=I1-IB2=114.1μA		I3=I2-IB1=107.2μA

Now, calculate the resistances and small signal parameters gm and rπ

RB1=VCC-VB2I1=76.8kΩ	RB2=VB2-VB1I2=43.8kΩ	RB3=VB1I3=53.2kΩ	      RE=VE1IE1=2.4kΩ

gm1=IC1VT=0.08S		rπ1=βgm1=3.6kΩ		gm2=IC2VT=0.08S	     rπ2=βgm2=3.6kΩ

Rin should be at least 5kΩ. Currently Rin=RB2RB3rπ1=3.12kΩ. It is necessary to add a resistor of at least R1 = 5-3.12=1.88kΩ at the input. Here are the standard values chosen for the resistors.

| RC | RB1 | RB2 | RB3 | RE | R1 |
| --- | --- | --- | --- | --- | --- |
| 2.4 kΩ | 75 kΩ | 43 kΩ | 51 kΩ | 2.4 kΩ | 2kΩ |

The low-f cut-in fL is specified to be at most 500 Hz. Calculate the low 3dB frequency point for the cascode amplifier using short circuit time constants. RBB=RB2||RB3=24kΩ, RL=50kΩ

τCEsc=CERE||rπ+RBB||Rs+R11+β=18.1CEyieldsωLp1=1τCEsc=118.1CE

τCC2sc=CC2RC+RL=52.4k∙CC2yieldsωLp2=1τCC2sc=152.4k∙CC2

τCC1sc=CC1Rs+R1+RBB||rπ=5.17k∙CC1yieldsωLp3=1τCC1sc=15.17k∙CC1

Notice that τCEsc≪τCC1scso do the open circuit time constant for CC1 instead.

τCC1oc=CC1Rs+R1+RBB||(rπ+(1+β)RE)=24.7k∙CC1yieldsωLp3=1τCC1sc=124.7k∙CC1

ωLz=1RECE=12.4k∙CE

Notice that ωLP1 is the dominant pole so ignore the other ones when calculating the 3dB frequency. CB is a large capacitor, so it does not need to be considered. Assume CE = CC1 = CC2.

ωL3dB=ωLp12-2ωLz2=117.8CE2-212.4k∙CE2=2πfL=1000π/s

yieldsCE=CC1=CC2=17.6μF

Calculate the midband gain |AV| > 50 as specified for the cascode amplifier.

Av=VoVπ2∙Vπ2Vπ1∙Vπ1Vs=-gmRC||RL∙1∙RB2RB3rπRB2RB3rπ+Rs+R1=-115

The midband voltage gain meets the specification. The standard values for the capacitors are selected.

| CE | CC1 | CC2 |
| --- | --- | --- |
| 22µF | 22µF | 22µF |

Finally, the full circuit can be constructed and the d.c operating point is simulated.

| VC2 | VB2 | VE2 | IC2 | IB2 | IE2 | VC1 | VB1 | VE1 | IC1 | IB1 | IE1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15.5V | 10.2V | 9.54V | 1.86mA | 14.2µA | 1.88mA | 9.54V | 5.2V | 4.54V | 1.88mA | 14.4µA | 1.89mA |

## Bode plots and estimated 3dB locations

fL3dB may be approximated using the dominant pole ωLp1. Use CE=22µF

ωLp1=118.1∙CE=3.139kradsyieldsfLp1=fL3dB=ωLp12π=500Hz

Apply miller’s theorem and then calculate the high 3dB frequency. VCB=VC2-VB2=VC1-VB1=4.3V.

cπ=2∙CJE+TF∙gm=42.4pF		cμ=CJC1+VCBVJCMJC=1.87pF

ωHp1=1cπ+2cμRBB|rπ|(R1+Rs)=17.5M rads

ωHp2=1cπ+2cμrπ1+β=1.81G rads

ωHp3=1cμRC||RL=234.1M rads

Notice the dominant high frequency pole is ωHp1 so the high 3dB point can be approximated using that.

fH3dB=ωHp12π=2.79MHz

|  | fL3dB | fH3dB |
| --- | --- | --- |
| calculated | 500Hz | 2.79MHz |
| simulated | 275.4Hz | 4.223MHz |

## Voltage Transfer curve

Set the source to the midband frequency of 100kHz. Vary the source amplitude from 0-0.5V.

The input voltage reaches about 0.025V before becoming non-linear.

## Input/Output Impedance measurement

Set the source to the midband frequency of 10kHz with an amplitude of 5mV. The input impedance can be calculated as Rin=RBB||rπ+R1=5.12kΩ.

To measure the input and output impedance remove the source resistor and short the load resistor. Use the AC RMS multimeter and probe the voltage & current directly at the source to measure Rin.

Rin=VrmsIrms=3.104mV584.2nA=5.313kΩ

The output impedance at midband is simply computed RC=2.4kΩ. To measure the output impedance use an AC multimeter to probe VC2 and IC2 with RL shorted.

Rout=VC2IC2=49.91μV20.80nA=2.4kΩ

## Discussion

The simulated results of the cascode amplifier meet the required specifications:

- Rout @ midband is within 2.5 kΩ ± 250Ω

- Rin > 5 kΩ @ midband

- |Av| > 50 @ midband

- fL3dB < 500Hz

However, the calculated and simulated 3dB points are significantly off. The calculated value of R1 =2kΩ was different from the actual required value of 3.3kΩ in simulation. I believe these inaccuracies could be caused by the fact that the beta used in the calculation was the ideal forward maximum beta but in simulation, it may have been significantly different. Nonetheless a cascode amplifier that meets the specifications is designed.

# Part 2 Cascaded Amplifiers

Design the repeater so that Rin & Rout are 50±5Ω and the low frequency 3dB point is less than 1kHz.

## Biasing

Here is the circuit at d.c. Use the 1/3 rule to bias.

VE1=VCC3=4V		VB1=VE1+VBE=4.7V

VC2=VCC=12V		VC1=VB2=23VCC=8V

VE2=VB2-VBE=7.3V	β=300

At the midband the input impedance is found to be

Rin=rπ1+β||RE1=β1+βVTIC1||VE1IE1=VTIE1||VE1IE1=VTIE1

50=25mVIE1yieldsIE1=IC1=500μA		IB1=IC1β=1.67μA			I1=0.1IE1=50μA

RB1=VCC-VB1I1=146kΩ			I3=I1-IB1=48.3μA		RB2=VB1I3=97.2kΩ

Rout=RE2||rπ2+RC11+β=RE2||RC11+β+VT∙RE2VE2=RE2||VCC-VC11+βIC1+VE2RE2+VT∙RE2VE2

50=RE2||VCC-VC11+βIC1+VE2RE2+VT∙RE2VE2yieldsRE2=7kΩ

IE2=VE2RE2=1.043mA		IB2=IE21+β=3.47μA		IC2=βIB2=1.040mA

I2=IC1+IB2=503μA

RC1=VCC-VC1IC2=3.85kΩ		RE1=VE1IE1=8kΩ		RBB=RB1||RB2=58.4kΩ

gm1=IC1VT=20mS		rπ1=βgm1=15kΩ		gm2=IC2VT=41.6mS	     rπ2=βgm2=7.2kΩ

Compute the fL3dB using the open and short circuit time constants.

τCC1sc=rπ11+β||RE1CC1=49.5CC1

τCBsc=(RBB|rπ1CB=11.9∙103CB

Notice τCC1sc≪τCBsc so the open circuit time constant for CB must be computed.

τCBoc=RBB||(rπ1+(1+β)RE1)CB=57∙103CB

τCC2sc=RE2||rπ2+RC11+βCC2=36.6CC2

Assume C = CC1 = CC2 and that they determine the low frequency cut-in. Assume CB is small and places a pole at least 1 decade away.

1000∙2πrads=1τCC1sc2+1τCC2sc2=149.5C2+136.6C2yieldsCC1=CC2=5.4μF

ωLp1=149.5∙5.4μF=3.734krads	 ωLp2=136.6∙5.4μF=5.059krads

ωL3dB=ωLp12+ωLp22=6.29krads=1.0006kHz

The pole associated with CB should be placed at least 1 decade below the 3dB point.

ωLp3=157∙103CB=6.29k10yieldsCB=27.9nF

Here are the standard values of the resistors and capacitors that will be used in the simulation.

| RE1 | RC1 | RB1 | RB2 | RE2 | CC1 | CB | CC2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8.2kΩ | 3.9kΩ | 150kΩ | 100kΩ | 6.8kΩ | 5.6µF | .027µF | 5.6µF |

The full cascaded amplifier is shown below.

## Measuring Input/Output Impedance & Midband Gain

To measure the input impedance, remove the source resistor, short the load resistor, and use a 1mV source with the midband frequency of 100kHz. Use an AC multimeter to measure the RMS voltage and current at the source node ‘A’.

Rin=VrmsIrms=702.6μV12.86μA=54.63Ω

To measure the output impedance, move the source to the output node and ground CC1. Then measure the RMS voltage and current at the node ‘A’ like before.

Rout=VrmsIrms=706.2μV12.89μA=54.79Ω

Both the input and output impedance is within 45-50Ω, the specification is met.

To measure the midband gain AM place the source back to the input side (like shown in the complete circuit of part A but with the load open circuited with a large resistance) and then observe the transient response to find the peak voltage at the output node ‘A’.

AM=44mV1mV=44

## Bode Plots

Add a 50Ω source and load resistance, the simulated circuit and its magnitude & phase bode plots are shown below.

| fL3dB | fH3dB |
| --- | --- |
| 832.4Hz | 4.802MHz |

## Discussion

The simulated results of the cascaded amplifier meet the required specifications:

- Rin & Rout @ midband is within 50Ω ± 5Ω

- The low frequency cut-in is below 1kHz

# Part 3 Cascaded Amplifiers

## Bode plots

Find the reference resistance. The current source should produce 2mA because the two emitter currents are specified to be 1mA.

12Io=IE1=IE2=1mA

Io=IREF1+2β=2mAyieldsIREF=2.01mA

IREF=0-(VEE+VBE)RRefyieldsRREF=7.1kΩ

## Calculating differential gain and fH3dB

Compute the high frequency 3dB point using open and short circuit time constants. VCB is measured to be approximately 3.3V at the d.c operating point.

gm=αIo2VT=40mS

cπ=2∙CJE+TF∙gm=25pF	cμ=CJC1+VCBVJCMJC=1.9pF		rπ=βgm=7.53kΩ

Ad=-2gmRCrπRS+2rπ=397VV

ωHp2=1cμ2∙2RC=52.54Mrads

The other pole is non-dominant so the 3dB point may be approximated by ωHp2

fH3dB=ωHp22π=8.36MHz

|  | fH3dB |
| --- | --- |
| Simulated (part A) | 4.245MHz |
| Calculated (part B) | 8.36MHz |

## Voltage Transfer curve

Choose a midband frequency of 100kHz and vary the peak amplitude of the source from 0.5mV to 50mV. Measure the output at the node ‘A’ with respect to ‘Ref’. Non-linear behavior starts happening at around 25mV.

## Discussion

The high frequency cut-off points calculated and simulated are substantially different, this could again be due to the β of 300.

# Part 4 Cascaded Amplifiers

## Differential Output

The simulated circuit and its differential output is shown below.

## Sine wave Saturation

Refer to the appendix for the plots of varying the input signal sine wave between 10mVp – 100mVp. Notice that distortion happens somewhere between 75mVp-85mVp. After testing some values between this range notice the largest input signal that results in an undistorted output signal is found to be 80mVp as shown below.

## Square wave Saturation

Refer to the appendix for the plots of varying the input signal square between 10mVp – 100mVp. Notice that distortion happens somewhere between 60mVp-70mVp. After testing some values between this range notice the largest input signal that results in an undistorted output signal is found to be approximately 65mVp as shown below.

## Discussion

The AM signal works by multiplying the signals together in the time domain (or convoluting them in the frequency domain). The modulated signal will consist of the two frequencies of the carrier and input, so the receiver can reconstruct the input signal by filtering for that specific frequency.

# References

- ELEC 301 Course Notes

- A. Sedra and K. Smith, "Microelectronic Circuits,"5 th (or higher) Ed., Oxford University Press, New York

- CircuitMaker™ User’s Manual

- 2N3904 datasheet [https://datasheetspdf.com/pdf-file/1114626/Motorola/2N3904/1]

- Standard Resistor and Capacitor Values

# Appendix

Part 4B: Varying the input voltage using a sin wave from 10mVp – 100mVp

10mVp:						20mVp:

30mVp:						40mVp:

60mVp:						75mVp:

85mVp:						100mVp:

Part 4C: Varying the input voltage using a square wave from 10mVp – 100mVp

10mVp:						20mVp:

35mVp:						50mVp:

60mVp:						70mVp:

85mVp:						100mVp: